Interpretation of Kt using simscape power system.

More poles generate more torque.

The contribution to the total torque from all phases are defined as:

Ta=KtIa⋅cosπ2-ΦR

Tb=KtIb⋅cos-π6-ΦR

Tc=KtIc⋅cos-5π6-ΦR

Where, Kt is the torque contant, ΦR is the instantaneous electrical angle of the rotor.

The total torque is the definded by:

T=Ta+Tb+Tc

Where N is the number of pole pairs.

With N being 4, and T_max is 38 Nm, Ta will be one sixth of that

Rotor angle definition is angle between q and a-phase magnetic axis

In the Simulink definition, Kt is divided by sqrt(3)*N… for some reason.

Current is then:

Tcont=1.5⋅In⋅Ke

and Ke is equal to the Ke as the :

Ke=Kt3⋅N≈0.0183

IMax=TMax⋅31.5⋅Kt

By this definition, the max current comes all the way up to 337.52 A, (at stall torque). This is proved by the fact that Ke=N*Kt for DC and PMSM motors, and that the line-voltage generated at a velocity corresponds to our measured voltage. The Kt version used in Simulink is Ke.