Rules for defining Kt and Ke

Ke=Ke=0,0733 Wb

Vp=Ke ωm

The back EMF voltage of any phase voltage is the product of Ke and the mechanical angular velocity

The torque contribution from each phase (regardless of number of pole pairs) is listed below.

Ta=KtIa⋅cosΦe-π2

Tb=KtIb⋅cosΦe-π2-23π

Tc=KtIc⋅cosΦe-π2+23π

Where Φe is the electrical angle of the rotor, and the currents depend on the angle of the rotor. The total torque produced is then:

T=Ta+Tb+Tc

If the magnetic field is 90 electrical degrees ahead of the permanent magnet, the torque can be reduced to:

T=KtI⋅1.5

Due to inductance of the motor, there’s a phase shift between the current and voltage waveforms. So these constants only apply when the primary side is only either current or voltage.

The peak current required to generate stall torque is then around 345 A

The maximum achievable velocity is 3971 RPM