Reasons for picking those component values

Packages will be 0806 wherever possible. Resistor values will be in the E96 series

Torque pedal:

R40 is 14k7. This will scale down the the voltage to a maximum of 5.1.

R41, R42 and C10 are decided by three equations:

R41+R42=54kΩ

VTP=R42sCR41R42+R41+R42⋅5.1= 1Vs=022s=2⋅π⋅20

Pedal switch utilizes a pull up resistor in the Zybo:

LEM sensors

ImaxNVmax=R

I max is 300, N is 2000, V max is 0.5. R is 3,33, so we round down to 3.

Non inverting summing amplifier:

Vout=1+R7R39⋅V1⋅R5R5+R6+V2⋅R6R5+R6

R7=3⋅R39

R7+R39=20 kΩ

Ideal full wave rectifier

Circuit developed like on this page: http://sound.westhost.com/appnotes/an001.htm

Filter calculated as

ωc=1R24⋅C3=2π30kHz

Schmitt Triggers

VCC-VhR18=VhR35+Vh-VrefR17

VDD-VlR18=VlR35+Vl-VrefR17

Vref = 2.5, vcc = 15, vee = 0.

Current through R17 must not exceed 100 uA.

Vh = 0.5 V

Vl = 0.4 V

Values selected are: R17=4k02, R18 = 95k3 and R35 = 768. This gives a turn-off threshold of 0.499 V, and a turn-on threshold of 0.398 V

Inverting comparator, reference signal.

Current should be limited, so these will be 10k each

N-channel Mosfet resistors

Voltage is always either 15 or 0, so a resistance of 10k.

ACD switch and En_gate resistors

R37 must be 7/3 the size of R38, R37 plus R38 must be twice the size of R36. This gives ADC switch 10V and En_gate 3.3 V.